GUIDANCE SHEET

Drinking Water and Groundwater Division

Instructions for Non-Acute Total Coliform Public Notice

Exceeding the total coliform bacteria maximum contaminant level is a Tier 1 violation.  You must provide public notice to persons served within 24 hours after you learn of the violation.  Within 24 hours, you must also contact the Division.  You should also coordinate with your local health officer.  The public notice must be reissued every 3 months for as long as the situation exists.  Use one or more of the following methods to deliver the notice to consumers:

	Radio

		Television

		Hand or direct delivery

	Posting in conspicuous locations (Community Water Systems may use posting as a secondary method, but must also use radio, television, or hand or direct delivery)

You may need to use additional methods (e.g., newspaper, delivery of multiple copies to hospitals, clinics, or apartment buildings), because notice must be provided in a manner expected to reach all persons served.

The notice on the reverse is appropriate for hand delivery or a newspaper notice. However, you may wish to modify it before using it for a radio or TV notice. If you do, you must still include all required elements and leave the health effects language in italics unchanged. This language is mandatory.  If you post or hand deliver, print your notice on letterhead, if you have it.

Description of the Violation

State the number of samples testing positive for coliform. The standard is that no more than one sample per month may be positive.

Corrective Action

In your notice, describe corrective actions you are taking. If you know what is causing the coliform problem, explain this in the notice. Listed below are some steps commonly taken by water systems with total coliform violations. Use one or more of the following actions, if appropriate, or develop your own:

		We are chlorinating and flushing the water system.

		We are increasing sampling for coliform bacteria.

		We are investigating the source of contamination.

		We are repairing the wellhead seal.

		We are repairing the storage tank.

		We will inform you when additional samples show no coliform bacteria.

After Issuing the Notice

Send a copy of each type of notice with the Certification section complete to the Division within ten days from the time you issue the notice.  The certification states that you have met all the public notice requirements.

It is a good idea to inform your customers when the violation is resolved.

This guidance sheet and related environmental information are available electronically via the internet. For information visit us through the Vermont Homepage at http://www.vermont.gov or visit DWGPD directly at www.drinkingwater.vt.gov

Drinking Water and Groundwater Protection Division

1 National Drive, Main Building, 2nd Floor

Montpelier, VT  05620-3521

Toll free 1-800-823-6500

Out of State 1-802-241-3400

Fax 1-802-828-1541

PUBLIC NOTICE

IMPORTANT INFORMATION ABOUT YOUR DRINKING WATER

Tests Show Coliform Bacteria in     [Insert System Name]     Water

Our water system recently violated a drinking water standard. Although this is not an emergency, as our customers, you have a right to know what happened, what you should do, and what we are doing to correct this situation.

We routinely monitor for the presence of drinking water contaminants. We took [insert # of samples] samples for coliform bacteria during [insert month].  [Insert #or percentage] of those samples showed the presence of coliform bacteria. The standard is that no more than [Insert “1 sample per month” or “5 percent of our samples”] may do so.

What should I do?

You do not need to boil your water or take other corrective actions. However, if you have specific health concerns, consult your doctor.

People with severely compromised immune systems, infants, and some elderly may be at increased risk. These people should seek advice about drinking water from their health care providers. General guidelines on ways to lessen the risk of infection by microbes are available from EPA’s Safe Drinking Water Hotline at 1 (800) 426-4791.

What does this mean?

This is not an emergency. If it had been you would have been notified immediately. Total coliform bacteria are generally not harmful themselves. Coliforms are bacteria which are naturally present in the environment and are used as an indicator that other, potentially-harmful, bacteria may be present. Coliforms were found in more samples than allowed and this was a warning of potential problems.

Coliforms are a sign that there could be a problem with the treatment or distribution system (pipes). Whenever we detect coliform bacteria in any sample, we do follow-up testing to see if other bacteria of greater concern, such as fecal coliform or E. coli, are present. We did not find any of these bacteria in our subsequent testing.

What happened? What is being done?

[Describe corrective action].

We anticipate resolving the problem within [estimated time frame].

For more information, please contact [name of contact] at [phone number] or [mailing address].

Please share this information with all the other people who drink this water, especially those who may not have received this notice directly (for example, people in apartments, nursing homes, schools, and businesses). You can do this by posting this notice in a public place or distributing copies by hand or mail.

Certification

Method(s) of Distribution:____________________________________________  Date Distributed:___________

(e.g. hand or direct delivery, posting¹, television, radio)

I _____________________(print name) Certify, as the Responsible Person (or authorized representative) of the water system indicated above, that the public notice has been provided to customers in accordance with the delivery, content, and format requirements and deadlines in the Vermont Water Supply Rule (Chapter 21, Subchapter 21-10).

Signature: __________________________		Date: _____________

Within 10 days of issuance of public notice, send a copy of the notice to:

Drinking Water and Groundwater Protection Division, One National Life Drive – Main 2, Montpelier, VT 05620-3521

1 - Community Water Systems may use posting as a second method, but must also use radio, television, or hand or direct delivery.